REDDUST
VOICE PATH ENGINEERING

From Airwaves to PCM, WebRTC, the RedDust Gateway, Syan and Back

A beginner-friendly engineering runbook for audio capture, digital audio, real-time networking and the end-to-end RedDust voice path

Based on: RedDust_WebRTC_Electronics_Communication_Kim.pptx

Project RedDust

From Anodiam

# Document Purpose and Scope

What this section does and why it matters Turn the 22-slide source deck into a complete engineering reference that an intern can read from the beginning without needing prior electronics, audio, networking or WebRTC knowledge. The document follows the journey of a spoken word from Kim's mouth, through the phone microphone and digital-audio pipeline, across WebRTC to the RedDust Gateway, onward to the Syan boundary, and back to Kim's speaker.

New technical concepts introduced in this section (in absolute beginner terms) This document connects four domains that are often taught separately: acoustics (sound in air), electronics (microphone and ADC), digital signal representation (sampling, quantisation and PCM), and real-time communications (WebRTC, RTP, ICE, STUN, TURN, DTLS and SRTP).

Why this matters specifically for RedDust RedDust is a real-time voice experience. A failure can occur at any one of these layers. Understanding the boundaries makes debugging much faster: microphone capture problems are different from codec problems, signaling problems, ICE problems, gateway media problems and playback problems.

Source-deck basis The first ten slides teach digital-audio sampling and PCM. Slides 11-22 then apply those ideas to Kim's voice, WebRTC, the RedDust Gateway, Syan, and the return path. This runbook preserves that teaching sequence and adds engineering detail where useful. Where the deck does not specify an implementation detail - especially the exact Gateway-to-Syan audio API - this document explicitly treats it as an architectural boundary rather than inventing a definitive implementation.

## Terminology Note: Mobile Front End, Gateway and Syan

What this section does and why it matters Remove ambiguity about which software component is being discussed.

New technical concepts introduced in this section (in absolute beginner terms) The mobile app is the user-facing front end. The RedDust Gateway is the backend WebRTC termination/bridging component. Syan is the conversational AI side of the architecture. Signaling and media are separate paths even though they cooperate to establish one voice session.

Why this matters specifically for RedDust When this document says 'Mobile App <-> Gateway', it means the WebRTC connection between the native mobile WebRTC stack and the Python/aiortc Gateway. When it says 'Gateway <-> Syan', it means the application-specific integration boundary beyond WebRTC.

| Term | Meaning in this runbook |
| --- | --- |
| Kim | The human caller/user in the source deck's example. |
| RedDust Mobile App | React Native/Expo application and the native WebRTC engine on the phone. |
| RedDust Gateway | Backend that handles signaling and terminates the WebRTC peer connection, using FastAPI + aiortc in the current project. |
| Syan | Conversational AI system that receives user audio/context and produces a response. The source deck does not define the exact transport/API between Gateway and Syan. |
| Signaling | Control messages used to negotiate a WebRTC session, e.g. SDP offer/answer. |
| Media | The real-time audio stream sent after connectivity is established. |

DIAGRAM PLACEHOLDER — A-01: End-to-End Voice Journey

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a landscape technical onboarding diagram titled 'RedDust Voice Journey - From Kim's Mouth to Syan and Back'. Show Kim speaking -> air pressure wave -> phone microphone -> pre-amplifier -> anti-aliasing filter -> ADC -> PCM/audio frames -> WebRTC audio processing/Opus -> RTP/SRTP over UDP -> Internet/NAT -> RedDust Gateway (FastAPI signaling + aiortc media) -> Syan integration boundary -> Syan response audio -> Gateway outbound WebRTC track -> SRTP/RTP -> mobile WebRTC jitter buffer/Opus decode -> DAC/audio amplifier -> speaker -> Kim. Use separate colors for physical acoustics, digital audio, signaling, media transport and application processing. Add a banner: 'Signaling sets up the call; WebRTC media carries the voice.'

# Contents

Part I - Sound, Electronics and Digital Audio Foundations

1. Sound in Air: The Analog Beginning

2. The Phone Microphone and Analog Electrical Signal

3. The ADC Capture Pipeline

4. Sampling: Discretising Time

5. Nyquist-Shannon Sampling Theorem

6. Aliasing and Anti-Aliasing Filters

7. Why 44.1 kHz Is Taught - and Why WebRTC Commonly Uses 48 kHz

8. Quantisation: Discretising Amplitude

9. Bit Depth, Noise Floor and Dynamic Range

10. PCM: Turning Samples into Numbers

11. WAV: A File Container Around PCM

Part II - From PCM to Real-Time WebRTC Media

12. Audio Frames and Buffers

13. Compression and the Opus Codec

14. Packetisation: RTP over UDP/IP

15. Jitter, Loss, Reordering and Real-Time Trade-offs

16. WebRTC as the Real-Time Communication Engine

17. Signaling versus Media

18. SDP Offer/Answer Negotiation

19. ICE, NAT, STUN and TURN

20. DTLS and SRTP Security

Part III - Exact RedDust Mobile-App-to-Gateway Flow

21. Mobile Capture and getUserMedia

22. RTCPeerConnection and Local Track Attachment

23. Offer Creation and ICE Gathering

24. FastAPI /offer Signaling

25. aiortc Answer Creation

26. Applying the Answer on the Mobile App

27. ICE Checks, DTLS and the Live Media Path

28. How the Gateway Receives Kim's Audio

29. Local Echo as a Proof of the Media Boundary

Part IV - Gateway, Syan and the Return Path

30. RedDust Gateway as the Bridge

31. Gateway-to-Syan Integration Boundary

32. Syan Hears, Understands and Responds

33. Building the Outbound Audio Track

34. The Return Network Path

35. Mobile Receive, Jitter Buffer, Decode and Speaker Playback

Part V - Engineering Calculations, Latency and Troubleshooting

36. Sampling and Frame Mathematics

37. Bitrate and Bandwidth Calculations

38. Latency Budget for Natural Conversation

39. Layer-by-Layer Troubleshooting

40. Security and Privacy Boundaries

41. Testing Strategy for RedDust Voice

42. Final End-to-End Mental Model

Appendix A - Glossary

Appendix B - Formula and Calculation Sheet

Appendix C - Diagnostic Checklist

Appendix D - Diagram Production Index

# Part I - Sound, Electronics and Digital Audio Foundations

The source presentation begins with a fundamental engineering problem: speech exists as a continuous physical wave, while software needs discrete numbers. The next sections explain every conversion between those two worlds.

# 1. Sound in Air: The Analog Beginning

What this section does and why it matters Explain what physically exists before any computer or phone processes the voice.

New technical concepts introduced in this section (in absolute beginner terms) Sound is a pressure disturbance travelling through a medium such as air. 'Analog' means the physical quantity can vary continuously rather than only taking fixed numerical steps.

Why this matters specifically for RedDust Kim's voice does not begin as PCM, Opus, packets or binary. It begins as moving air. Every later digital representation is an engineered description of that original physical event.

Source-deck basis Slides 2 and 12 describe true sound as a continuous pressure wave and show Kim's speech becoming an analog electrical signal at the microphone.

When Kim speaks, her vocal cords and vocal tract cause nearby air pressure to vary. The pressure rises above and falls below the surrounding atmospheric pressure. Those variations propagate outward as sound waves. A microphone does not 'hear words'; it responds to these tiny pressure changes.

## 1.1 Frequency, amplitude and phase

What this section does and why it matters Introduce three measurements used to describe a waveform.

New technical concepts introduced in this section (in absolute beginner terms) Frequency describes how rapidly a repeating component changes, amplitude describes its strength, and phase describes where a repeating cycle is relative to another reference.

Why this matters specifically for RedDust Voice contains many frequencies at the same time. The phone microphone captures the combined waveform, not a single perfect sine wave.

• Frequency is measured in hertz (Hz), meaning cycles per second. Higher-frequency components are generally perceived as higher pitch or fine detail.

• Amplitude represents the size of the pressure variation. In a microphone circuit, that becomes the size of a changing voltage.

• A real spoken syllable is a complex sum of many frequency components whose amplitudes change continuously over time.

DIAGRAM PLACEHOLDER — A-02: Analog Speech Wave

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a clean beginner diagram titled 'Kim Speaks - The Analog Beginning'. Show Kim's vocal cords -> changing air pressure -> waveform in air -> phone microphone diaphragm. Add axes for time and pressure and annotations for amplitude and frequency. Include the statement: 'The physical wave is continuous in time and amplitude.'

# 2. The Phone Microphone and Analog Electrical Signal

What this section does and why it matters Explain how a physical pressure wave becomes an electrical quantity that electronics can process.

New technical concepts introduced in this section (in absolute beginner terms) A microphone is a transducer: a device that converts energy from one form into another. Here it converts acoustic pressure into an analog electrical signal.

Why this matters specifically for RedDust The RedDust app cannot directly read air pressure. The phone hardware first converts the acoustic event into an electrical representation.

The microphone diaphragm moves in response to air pressure. Internal microphone electronics convert that motion into a very small electrical signal. The voltage follows the shape of the acoustic waveform closely enough to preserve the information needed for later digitisation.

## 2.1 Why the signal must be conditioned

What this section does and why it matters Explain why microphone voltage is not fed blindly into digital logic.

New technical concepts introduced in this section (in absolute beginner terms) Signal conditioning includes amplification and filtering before analog-to-digital conversion.

Why this matters specifically for RedDust Good capture begins in hardware. Software cannot fully repair clipping, severe analog noise or aliasing that was introduced before the samples were created.

• The raw microphone signal is very small and needs amplification to use the ADC's available input range effectively.

• The analog path must avoid clipping. If amplification is too high, waveform peaks hit the hardware limit and flatten.

• The analog path must also limit frequencies that the chosen digital sample rate cannot represent safely.

DIAGRAM PLACEHOLDER — A-03: Microphone as a Transducer

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a landscape diagram showing sound pressure -> diaphragm motion -> tiny analog voltage -> pre-amplifier. Use a side-by-side pressure waveform and voltage waveform to show that the information is preserved while the physical quantity changes.

# 3. The ADC Capture Pipeline

What this section does and why it matters Walk through the complete audio-capture hardware pipeline shown in the presentation.

New technical concepts introduced in this section (in absolute beginner terms) ADC means Analog-to-Digital Converter. Before the ADC, the signal is continuous voltage. After the ADC, the audio can be represented as a sequence of numerical samples.

Why this matters specifically for RedDust This pipeline is the boundary between physics/electronics and software. Once the samples exist, software and the WebRTC engine can process them.

Source-deck basis Slide 3 presents: sound pressure -> microphone voltage -> pre-amplification -> anti-aliasing low-pass filter -> ADC sampling/quantisation -> PCM numbers -> WAV container. Slides 13-16 apply the same sequence to Kim's phone.

| Stage | Beginner explanation | Failure symptom |
| --- | --- | --- |
| Microphone | Turns sound pressure into a tiny electrical waveform. | No/poor capture, microphone hardware issue. |
| Pre-amplifier | Boosts the weak signal to a useful level. | Too quiet, noisy, or clipped if gain is wrong. |
| Anti-aliasing filter | Removes frequencies that would be unsafe to sample. | Irreversible aliasing if insufficient. |
| ADC | Measures the waveform at fixed times and maps each measurement to a numeric level. | Wrong sample rate/bit depth or hardware distortion. |
| PCM/audio frames | Software-readable sample values arranged in time. | Format/channel/rate mismatch in later processing. |

DIAGRAM PLACEHOLDER — A-04: ADC Pipeline

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a presentation-ready diagram titled 'Audio Capture and Digital Conversion Pipeline'. Show seven numbered stages: sound pressure, microphone voltage, pre-amplifier, anti-aliasing filter, ADC sampling + quantisation, PCM numbers, and optional WAV container. Clearly mark the analog domain before ADC and digital domain after ADC.

# 4. Sampling: Discretising Time

What this section does and why it matters Explain the first half of digitisation: choosing when to measure the analog waveform.

New technical concepts introduced in this section (in absolute beginner terms) Sampling converts continuous time into discrete time. The sample rate fs is how many measurements are taken each second; the sampling interval Ts is the time between measurements, with Ts = 1/fs.

Why this matters specifically for RedDust A WebRTC audio pipeline ultimately operates on blocks made from these samples. Understanding sample rate is necessary for frame sizing, codec timing and resampling.

Core sampling relationship

x[n] = x(n * Ts) Ts = 1 / fs

If fs = 48,000 samples/second, the ADC or digital capture pipeline produces one sample period every 1/48,000 second, approximately 20.83 microseconds per sample per channel.

## 4.1 What a sample is - and is not

What this section does and why it matters Prevent the common misconception that one sample is a tiny chunk of audio.

New technical concepts introduced in this section (in absolute beginner terms) A sample is one numeric amplitude measurement at one instant. A frame or buffer is a collection of many samples.

Why this matters specifically for RedDust WebRTC processes frames/buffers; the underlying information is still built from individual samples.

Important caution Do not imagine digital audio as a permanently jagged staircase that the speaker reproduces literally. Proper band-limited reconstruction produces a smooth waveform. The sample points contain enough information to reconstruct the band-limited signal when the sampling theorem is satisfied.

DIAGRAM PLACEHOLDER — A-05: Sampling in the Time Domain

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a graph-based diagram titled 'Sampling - Discretising Time'. Draw a smooth analog waveform and uniformly spaced sample markers. Label fs, Ts, x(t), and x[n] = x(nTs). Add a small 48 kHz example showing 48,000 measurements per second.

# 5. Nyquist-Shannon Sampling Theorem

What this section does and why it matters Explain the mathematical rule that determines how fast a signal must be sampled.

New technical concepts introduced in this section (in absolute beginner terms) For a band-limited signal, the sampling rate must be greater than twice the highest frequency that must be represented. The Nyquist frequency is fs/2.

Why this matters specifically for RedDust This rule explains why audio systems sample tens of thousands of times per second even though humans perceive frequencies only up to roughly 20 kHz.

Nyquist-Shannon rule (engineering form)

fs > 2 * fmax Nyquist frequency = fs / 2

The theorem is not saying 'two samples make a nice-looking wave'. It is a frequency-domain statement: if the signal contains no energy above the Nyquist limit and the system samples correctly, the original band-limited waveform is mathematically reconstructable from the samples.

## 5.1 Human hearing and practical engineering margin

What this section does and why it matters Connect the theorem to audio sample-rate choices.

New technical concepts introduced in this section (in absolute beginner terms) Human hearing is commonly approximated as extending to about 20 kHz for a young healthy listener. Real anti-aliasing filters need a transition band, so practical sample rates are above the bare 40 kHz mathematical minimum.

Why this matters specifically for RedDust The source deck uses 44.1 kHz as the familiar teaching example. RedDust's WebRTC path must also understand the 48 kHz world used by Opus/WebRTC.

DIAGRAM PLACEHOLDER — A-06: Nyquist Frequency and Audio Band

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a frequency-domain diagram titled 'Nyquist-Shannon for Audio'. Show audible band to ~20 kHz, a low-pass transition band, Nyquist frequency, and example sample rates 44.1 kHz and 48 kHz. Explain that frequencies above fs/2 must not enter the sampled signal.

# 6. Aliasing and Anti-Aliasing Filters

What this section does and why it matters Explain what goes wrong when the sampling rule is violated and why a low-pass filter sits before the ADC.

New technical concepts introduced in this section (in absolute beginner terms) Aliasing is the false appearance of a high-frequency signal as a different lower frequency after under-sampling. Once it is encoded into the digital samples, it cannot be cleanly separated from genuine low-frequency content.

Why this matters specifically for RedDust The anti-aliasing filter is a hardware protection mechanism. It keeps out frequencies that the chosen sample rate cannot represent safely.

Source-deck basis Slide 6 describes under-sampling, spectral folding, permanent digital distortion and prevention by the hardware anti-aliasing filter.

A useful mental model is a camera filming a spinning wheel. If frames are captured too slowly, the wheel may appear to turn slowly backward. The motion was not actually backward; the sampling process created an alias. Audio aliasing is the same category of error in the frequency domain.

## 6.1 Why software cannot simply fix it later

What this section does and why it matters Explain the irreversibility of aliasing.

New technical concepts introduced in this section (in absolute beginner terms) After aliasing, the false frequency occupies the same digital representation as a legitimate lower frequency. The original high-frequency identity has been lost.

Why this matters specifically for RedDust Debugging must distinguish aliasing from codec artifacts, packet loss and jitter. They sound different and originate at different layers.

DIAGRAM PLACEHOLDER — A-07: Aliasing Before and After

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a two-panel diagram. Top: adequate sampling of a high-frequency sine wave. Bottom: under-sampling causing an apparent lower-frequency alias. Add an anti-aliasing low-pass filter before the ADC as the prevention step.

# 7. Why 44.1 kHz Is Taught - and Why WebRTC Commonly Uses 48 kHz

What this section does and why it matters Reconcile the source presentation's 44.1 kHz examples with the RedDust WebRTC implementation context.

New technical concepts introduced in this section (in absolute beginner terms) 44.1 kHz is a classic digital-audio/CD sample rate. 48 kHz is a common professional video, telephony and WebRTC audio rate. A system can resample between rates when required.

Why this matters specifically for RedDust Interns must not assume that because a WAV teaching example uses 44.1 kHz, every RedDust WebRTC frame or Opus timestamp uses 44.1 kHz.

Source-deck basis Slides 7 and 14 use 44.1 kHz to explain human hearing and sampling. That teaching example is retained here.

Engineering elaboration beyond the deck For WebRTC voice, Opus uses a 48 kHz RTP clock and WebRTC implementations commonly process audio on a 48 kHz timeline, even when the physical device hardware has its own native capture rate. The WebRTC/native audio stack may resample internally. Therefore the correct engineering rule is: know the rate at each boundary; do not assume one rate survives unchanged end-to-end.

| Context | Typical rate used in explanation | What to remember |
| --- | --- | --- |
| CD / classic stored audio | 44.1 kHz | Useful Nyquist and PCM teaching example. |
| Professional/video audio | 48 kHz | Very common system rate. |
| WebRTC / Opus media clock | 48 kHz | Important for frame timing and RTP timestamps. |
| Phone hardware | Device-dependent | Native layer may resample before/inside WebRTC. |

DIAGRAM PLACEHOLDER — A-08: 44.1 kHz versus 48 kHz Boundaries

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a beginner diagram comparing a 44.1 kHz WAV teaching path with a 48 kHz WebRTC/Opus path. Show that resampling can occur and annotate: 'Do not assume the phone's hardware rate equals the network media clock.'

# 8. Quantisation: Discretising Amplitude

What this section does and why it matters Explain the second half of digitisation: mapping each measured voltage to one of a finite set of numeric values.

New technical concepts introduced in this section (in absolute beginner terms) Sampling chooses WHEN to measure; quantisation chooses WHICH numeric amplitude value represents that measurement. Bit depth determines how many possible amplitude levels exist.

Why this matters specifically for RedDust Quantisation determines the precision/noise floor of PCM representations used in processing, storage and intermediate audio frames.

For an N-bit linear PCM representation, there are 2^N possible codes. A 16-bit representation has 65,536 possible codes; a 24-bit representation has 16,777,216. More levels mean smaller rounding steps for a given full-scale range.

Bit-depth relationship

Quantisation levels = 2^N 16-bit -> 65,536 levels 24-bit -> 16,777,216 levels

## 8.1 Quantisation error

What this section does and why it matters Explain why the digitised value is usually not exactly equal to the original analog voltage.

New technical concepts introduced in this section (in absolute beginner terms) The ADC rounds each measurement to the nearest available code. The difference is quantisation error; across audio it behaves like a low-level noise component under normal assumptions.

Why this matters specifically for RedDust Quantisation noise is different from microphone noise, codec loss and network loss. Those are separate sources of degradation.

DIAGRAM PLACEHOLDER — A-09: Quantisation Grid

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a graph titled 'Quantisation - Discretising Amplitude'. Show a smooth sampled waveform, horizontal amplitude levels, rounded sample values and arrows identifying quantisation error. Compare a coarse low-bit example with a fine 16-bit example.

# 9. Bit Depth, Noise Floor and Dynamic Range

What this section does and why it matters Connect bit depth to usable signal range and numerical precision.

New technical concepts introduced in this section (in absolute beginner terms) Dynamic range is the span between very quiet and very loud representable signals. For ideal linear PCM, each additional bit adds roughly 6 dB of theoretical signal-to-quantisation-noise ratio.

Why this matters specifically for RedDust The important RedDust lesson is to preserve sensible headroom and avoid clipping. More bits do not repair an already clipped analog signal.

Engineering elaboration beyond the deck A widely used ideal approximation for a full-scale sine wave in an N-bit quantiser is SNR ≈ 6.02N + 1.76 dB. Real devices also contain analog self-noise, microphone noise and other limitations, so the theoretical number is not the whole system's real-world dynamic range.

| Bit depth | Number of levels | Idealized implication |
| --- | --- | --- |
| 8-bit | 256 | Very coarse for high-quality linear PCM. |
| 16-bit | 65,536 | Common consumer PCM precision; about 96 dB idealized quantisation SNR. |
| 24-bit | 16,777,216 | Much finer recording/processing headroom; about 144 dB idealized quantisation SNR. |

Important caution Bit depth and codec bitrate are not the same thing. '16-bit PCM' describes uncompressed sample precision. 'Opus at 24 kbps' describes a compressed network bitrate. They belong to different layers.

DIAGRAM PLACEHOLDER — A-10: Bit Depth and Dynamic Range

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create an infographic showing 8-bit, 16-bit and 24-bit amplitude grids, quantisation step size, noise floor and headroom. Add a warning that clipping is an analog/digital full-scale problem and is not fixed by choosing more bits after the fact.

# 10. PCM: Turning Samples into Numbers

What this section does and why it matters Explain Pulse Code Modulation as the basic numerical representation of sampled/quantised audio.

New technical concepts introduced in this section (in absolute beginner terms) PCM is a time-ordered stream of numeric sample values. It is not inherently a file format and it is not a network protocol.

Why this matters specifically for RedDust Inside media processing code, decoded audio is often handled in PCM-like sample frames even when the network transport uses compressed Opus packets.

Source-deck basis Slide 9 describes quantised integers being represented in binary and sent sequentially; slide 16 connects PCM to WAV and notes that RedDust's audio frames are packetised for network transport.

A PCM stream needs context: sample rate, sample format/bit depth, number of channels and sample ordering. Without that context, a sequence of bytes is ambiguous.

## 10.1 Signed integer representation

What this section does and why it matters Explain why audio samples can be positive and negative while computers store bits.

New technical concepts introduced in this section (in absolute beginner terms) Linear PCM often uses signed integers. Negative waveform amplitudes are represented using a signed binary scheme such as two's complement.

Why this matters specifically for RedDust Libraries such as PyAV expose decoded frames and formats so application code does not need to hand-convert every binary sample.

PCM sample sequence

Example conceptual sample sequence: +1200, +900, +250, -400, -1100, ... Each value corresponds to one instant on the waveform timeline.

DIAGRAM PLACEHOLDER — A-11: Quantised Samples to PCM

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a left-to-right diagram: quantised signed sample values -> binary two's-complement representation -> sequential PCM stream. Label sample rate, bit depth and channel count as metadata required to interpret the stream.

# 11. WAV: A File Container Around PCM

What this section does and why it matters Separate the idea of PCM sample data from the idea of storing that data in a file.

New technical concepts introduced in this section (in absolute beginner terms) WAV is a container format. A WAV file typically has header chunks describing the audio format and a data chunk containing audio sample bytes. PCM is the underlying sample representation in the common uncompressed case.

Why this matters specifically for RedDust RedDust's live WebRTC media is not sent as a WAV file. WAV is useful for assets, recordings and tests; live WebRTC uses media frames, codecs and packets.

| WAV component | Purpose |
| --- | --- |
| Container/header chunks | Identify format, sample rate, channels, sample representation and sizes. |
| Data chunk | Contains the audio sample bytes for uncompressed PCM WAV. |
| Not a live transport | A WAV file is not what WebRTC sends packet-by-packet across the network. |

Source-deck CD-quality example

Uncompressed PCM bit rate = sample_rate * bits_per_sample * channels 44,100 * 16 * 2 = 1,411,200 bits/s = 1,411.2 kbps

DIAGRAM PLACEHOLDER — A-12: PCM versus WAV

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a diagram titled 'PCM Is Data; WAV Is a Container'. Show raw PCM samples entering a WAV box with a header chunk and data chunk. Beside it show a separate live WebRTC path: PCM frame -> Opus -> RTP/SRTP -> network. Emphasize that WebRTC does not transmit a WAV file.

# Part II - From PCM to Real-Time WebRTC Media

Once audio exists as digital samples, RedDust still needs to move it quickly, securely and robustly across changing networks. This requires framing, compression, packetisation, connectivity discovery and encryption.

# 12. Audio Frames and Buffers

What this section does and why it matters Explain how software groups individual samples into manageable blocks for real-time processing.

New technical concepts introduced in this section (in absolute beginner terms) A frame is a time-bounded block of audio samples. A buffer is memory used to hold one or more frames while components produce and consume audio at slightly different times.

Why this matters specifically for RedDust Latency in RedDust is strongly affected by frame duration and buffering. Very large buffers are stable but feel sluggish; very small buffers reduce latency but leave less tolerance for scheduling/network variation.

Engineering elaboration beyond the deck A common illustrative WebRTC/Opus frame duration is 20 ms. At 48 kHz, 20 ms corresponds to 960 samples per channel: 48,000 samples/s * 0.020 s = 960 samples. Actual implementations may use other frame sizes internally.

Illustrative frame calculation

48,000 samples/s * 0.020 s = 960 samples per channel 50 frames/s when each frame is 20 ms

DIAGRAM PLACEHOLDER — B-01: Samples to Frames to Buffers

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a timeline diagram showing individual PCM samples grouped into 20 ms audio frames and frames held briefly in buffers. Annotate the trade-off between low latency and resilience to scheduling/network jitter.

# 13. Compression and the Opus Codec

What this section does and why it matters Explain why live voice is not normally sent as raw PCM across the Internet.

New technical concepts introduced in this section (in absolute beginner terms) A codec is an encoder/decoder. Opus compresses PCM-like audio into a much smaller representation for transmission and decodes received compressed audio back into audio samples.

Why this matters specifically for RedDust WebRTC commonly negotiates Opus for audio. React Native and aiortc normally let the WebRTC stacks perform codec work automatically; RedDust application code should not manually encode/decode ordinary WebRTC media unless there is a special processing requirement.

Raw PCM can consume more than a megabit per second for stereo CD-quality audio. Conversational speech can be represented far more efficiently by a perceptual codec such as Opus, reducing network load while keeping latency low.

| PCM | Opus |
| --- | --- |
| Uncompressed numerical samples. | Compressed audio bitstream. |
| Predictable high data rate. | Much smaller bitrate, often adaptive. |
| Convenient for DSP/frame processing. | Convenient for network transport. |
| No codec loss by itself. | Lossy compression designed for perceptual quality and real-time use. |

DIAGRAM PLACEHOLDER — B-02: PCM to Opus and Back

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a symmetric diagram: microphone PCM frames -> Opus encoder -> compact Opus frames -> network -> Opus decoder -> PCM frames -> speaker. Place 'native WebRTC stack' around the codec boxes and show that React Native application code normally does not implement the codec itself.

# 14. Packetisation: RTP over UDP/IP

What this section does and why it matters Explain how compressed media is divided into network packets and given timing/ordering metadata.

New technical concepts introduced in this section (in absolute beginner terms) RTP (Real-time Transport Protocol) carries real-time media metadata such as sequence numbers and timestamps. UDP (User Datagram Protocol) carries datagrams without retransmission guarantees. IP routes packets between network addresses.

Why this matters specifically for RedDust For live conversation, timeliness matters more than perfect delivery. A packet arriving far too late may be less useful than a packet that is simply lost.

Source-deck basis Slide 17 shows digital audio frames being split into small packets that travel independently over the Internet.

The practical stack is layered. Opus describes compressed audio content. RTP describes how real-time media units are timestamped/sequenced. SRTP encrypts/authenticates the RTP media. UDP provides the low-overhead datagram transport. IP moves the datagrams across networks.

Conceptual outbound layering

Audio samples -> Opus encoded frame -> RTP packet -> SRTP protection -> UDP datagram -> IP packet -> network

DIAGRAM PLACEHOLDER — B-03: WebRTC Media Packet Stack

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a layered packet diagram titled 'How One Audio Frame Becomes a Network Packet'. Show PCM -> Opus -> RTP header+payload -> SRTP encrypted packet -> UDP -> IP -> Ethernet/Wi-Fi/5G. On receive, reverse the stack.

# 15. Jitter, Loss, Reordering and Real-Time Trade-offs

What this section does and why it matters Explain why Internet packets do not arrive with perfect timing and what the receiver does about it.

New technical concepts introduced in this section (in absolute beginner terms) Jitter means variation in packet arrival time. Packet loss means some packets never arrive. Reordering means packets can arrive in a different order from the one in which they were sent.

Why this matters specifically for RedDust WebRTC's receive pipeline uses sequence numbers, timestamps, jitter buffering, codec resilience and other mechanisms to turn an imperfect packet stream into intelligible audio without adding excessive delay.

A jitter buffer intentionally waits a small amount of time so slightly early and late packets can be played in the correct timeline. Increasing the buffer can hide more network variation, but it increases conversational delay. This is one of the fundamental real-time trade-offs.

| Network condition | Real-time consequence | Typical mitigation concept |
| --- | --- | --- |
| Jitter | Uneven playback timing. | Adaptive jitter buffer. |
| Packet loss | Missing audio information. | Codec packet-loss concealment / FEC where available; do not wait indefinitely. |
| Reordering | Packets arrive out of sequence. | RTP sequence numbers/timestamps allow reordering within useful time. |
| Long delay | Conversation feels unnatural. | Keep buffers and processing bounded; choose low-latency paths. |

DIAGRAM PLACEHOLDER — B-04: Jitter Buffer

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a timeline diagram showing packets sent at regular intervals but arriving early/late/out-of-order. Show the jitter buffer reordering and pacing them into smooth playback, with a latency-versus-resilience slider.

# 16. WebRTC as the Real-Time Communication Engine

What this section does and why it matters Explain WebRTC as a coordinated system rather than a single protocol.

New technical concepts introduced in this section (in absolute beginner terms) WebRTC combines media capture, codecs, RTP, connectivity establishment, NAT traversal, congestion adaptation and encryption. It also exposes peer-connection APIs to applications.

Why this matters specifically for RedDust RedDust relies on native WebRTC on the mobile side and aiortc on the Python side so the team does not implement RTP, ICE, DTLS, SRTP and codecs from scratch.

Source-deck basis Slide 18 lists peer-to-peer connection, NAT traversal (ICE/STUN/TURN), secure encryption (DTLS/SRTP), adaptive quality, low latency and jitter control as WebRTC capabilities.

| WebRTC responsibility | RedDust implication |
| --- | --- |
| Media capture/track model | Microphone appears as a MediaStreamTrack. |
| Codec negotiation | Opus and other capabilities described in SDP. |
| Connectivity | ICE searches for a working path. |
| NAT traversal | STUN and, when required, TURN help cross network boundaries. |
| Security | DTLS establishes keying; SRTP protects media. |
| Timing/adaptation | Jitter and network changes are handled in the media engine. |

DIAGRAM PLACEHOLDER — B-05: WebRTC Engine Responsibilities

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a hub-and-spoke diagram with WebRTC Engine in the center and spokes for media capture, codec, RTP, ICE, STUN/TURN, DTLS, SRTP, congestion adaptation, jitter buffer, track events and statistics. Mark application code outside the engine.

# 17. Signaling versus Media

What this section does and why it matters Make the most important architectural distinction in the RedDust voice path.

New technical concepts introduced in this section (in absolute beginner terms) Signaling exchanges control information used to establish the WebRTC session. Media is the continuous encrypted audio flow after the peer connection is negotiated. WebRTC intentionally does not mandate one signaling transport.

Why this matters specifically for RedDust In the current RedDust local echo implementation, FastAPI HTTP POST /offer is signaling. The microphone audio itself does not stream through that HTTP POST; it flows through WebRTC/aiortc media transport.

Two separate paths

SIGNALING PATH Mobile app --HTTP POST /offer--> FastAPI <--SDP answer-------- MEDIA PATH Native WebRTC == SRTP/RTP/UDP == aiortc audio track <----------> audio track

Important caution Do not debug a media problem only by looking at the FastAPI 200 OK. A successful /offer proves signaling reached the Gateway, not that ICE connected or that audio packets are flowing.

DIAGRAM PLACEHOLDER — B-06: Signaling Path versus Media Path

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a split-lane diagram. Top lane: Mobile TypeScript -> HTTP POST /offer -> FastAPI -> aiortc SDP answer -> Mobile. Bottom lane: Native WebRTC -> ICE/DTLS/SRTP/UDP -> aiortc media -> audio track. Label top 'control plane/signaling' and bottom 'media plane'.

# 18. SDP Offer/Answer Negotiation

What this section does and why it matters Explain the description exchange that lets two WebRTC peers agree on media and transport parameters.

New technical concepts introduced in this section (in absolute beginner terms) SDP (Session Description Protocol) is a text description. It can state media types, codecs, ICE credentials, DTLS fingerprint information, direction attributes and candidates. SDP is not itself the media transport.

Why this matters specifically for RedDust The RedDust mobile app creates an offer; aiortc accepts it as the remote description, creates an answer, and the phone applies that answer as its remote description.

Offer-answer sequence

Mobile Gateway RTCPeerConnection RTCPeerConnection ¦ createOffer() ¦ ¦ setLocalDescription() ¦ ¦------ SDP OFFER via HTTP --------------->¦ ¦ setRemoteDescription() ¦ createAnswer() ¦ setLocalDescription() ¦<----- SDP ANSWER via HTTP ---------------¦ setRemoteDescription(answer) ¦ ICE connectivity checks begin / continue

## 18.1 What interns should recognize in SDP

What this section does and why it matters Teach recognition without encouraging manual SDP editing.

New technical concepts introduced in this section (in absolute beginner terms) Lines such as m=audio, a=rtpmap, a=ice-ufrag, a=fingerprint and a=candidate describe different aspects of the session.

Why this matters specifically for RedDust Logs can use SDP as evidence, but application code should let the WebRTC implementation generate and parse it.

| SDP item | Beginner meaning |
| --- | --- |
| m=audio | There is an audio media section. |
| a=rtpmap:... opus/... | Opus is offered/negotiated for a payload type. |
| a=sendrecv / recvonly | Media direction capability. |
| a=ice-ufrag / a=ice-pwd | Temporary ICE credentials. |
| a=fingerprint | DTLS certificate fingerprint used for secure transport setup. |
| a=candidate | A possible network address/path for ICE to test. |

DIAGRAM PLACEHOLDER — B-07: SDP Offer/Answer

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a sequence diagram showing mobile createOffer/setLocalDescription -> HTTP POST /offer -> aiortc setRemoteDescription/createAnswer/setLocalDescription -> JSON answer -> mobile setRemoteDescription. Include callouts showing examples of codec, ICE and DTLS information contained in SDP.

# 19. ICE, NAT, STUN and TURN

What this section does and why it matters Explain how the peers discover a usable network path when phones and servers sit behind routers, firewalls and changing networks.

New technical concepts introduced in this section (in absolute beginner terms) NAT translates private addresses to public-facing addresses. ICE (Interactive Connectivity Establishment) gathers candidate addresses and tests candidate pairs. STUN helps a peer discover a server-reflexive public mapping. TURN relays media when a direct path cannot be established.

Why this matters specifically for RedDust The local same-Wi-Fi echo test can often succeed with host candidates. Internet/mobile networks make TURN much more important for production reliability.

Source-deck basis Slide 18 explicitly names NAT, ICE, STUN and TURN as the NAT-traversal components of the WebRTC engine.

| Term | Full form | Role |
| --- | --- | --- |
| NAT | Network Address Translation | Maps private addresses/ports to public-facing mappings. |
| ICE | Interactive Connectivity Establishment | Collects candidates and tests which path works. |
| STUN | Session Traversal Utilities for NAT | Helps discover the public mapping visible from outside. |
| TURN | Traversal Using Relays around NAT | Relays traffic when direct candidate pairs cannot work. |

DIAGRAM PLACEHOLDER — B-08: ICE/STUN/TURN Candidate Discovery

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a network diagram with phone behind home/mobile NAT, Gateway on another network, STUN server and TURN server. Show host, server-reflexive and relay candidates; then show ICE testing candidate pairs and selecting the best working path.

# 20. DTLS and SRTP Security

What this section does and why it matters Explain how WebRTC protects media in transit.

New technical concepts introduced in this section (in absolute beginner terms) DTLS (Datagram Transport Layer Security) performs a secure handshake over the datagram path and establishes keying material. SRTP (Secure Real-time Transport Protocol) protects the RTP audio packets using those keys.

Why this matters specifically for RedDust The RedDust TypeScript and Python application layers should not manually encrypt/decrypt normal WebRTC audio. The WebRTC stacks perform these security operations.

The media path is therefore not 'plain UDP audio'. UDP is only the transport underneath. The RTP media is protected as SRTP, and key establishment/authentication is tied to the WebRTC DTLS handshake and SDP fingerprints.

Protected media path

RTP media -> SRTP encryption/authentication -> UDP/IP -> Internet -> UDP/IP -> SRTP verification/decryption -> RTP media

DIAGRAM PLACEHOLDER — B-09: DTLS-SRTP Security

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a two-peer diagram. Show SDP fingerprints exchanged during signaling, DTLS handshake over the selected ICE path, keys established, then SRTP-protected audio packets flowing in both directions. Add banner: 'Application code does not manually encrypt/decrypt normal WebRTC media.'

# Part III - Exact RedDust Mobile-App-to-Gateway Flow

This part turns the general WebRTC concepts into the concrete RedDust local architecture used in the Steps 0-19 echo implementation: React Native + react-native-webrtc on the phone, FastAPI for signaling, and aiortc/PyAV for the Gateway media peer.

# 21. Mobile Capture and getUserMedia

What this section does and why it matters Explain exactly where Kim's microphone enters the RedDust application.

New technical concepts introduced in this section (in absolute beginner terms) getUserMedia() asks the native WebRTC layer for access to a capture device and returns a MediaStream containing MediaStreamTrack objects. The JavaScript layer receives track objects, not raw microphone voltages.

Why this matters specifically for RedDust The operating system and native WebRTC stack handle the hardware capture pipeline. React Native code requests and orchestrates the media rather than reading the ADC directly.

Conceptual mobile capture call

const localStream = await mediaDevices.getUserMedia({ audio: true, video: false, });

After permission is granted, the local stream typically contains one live audio track. That track is a high-level handle to the ongoing native audio source.

DIAGRAM PLACEHOLDER — C-01: Mobile Audio Capture Boundary

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a layered phone diagram: microphone hardware -> preamp/filter/ADC -> mobile OS audio subsystem -> native WebRTC audio capture -> react-native-webrtc MediaStream -> TypeScript application. Clearly label which layers are hardware/native and which are JavaScript.

# 22. RTCPeerConnection and Local Track Attachment

What this section does and why it matters Show how the captured microphone is attached to the object that represents the remote WebRTC session.

New technical concepts introduced in this section (in absolute beginner terms) RTCPeerConnection is the main WebRTC connection object. A local audio track must be attached before the offer is generated so the SDP can describe the intention to send audio.

Why this matters specifically for RedDust In RedDust, the phone's PeerConnection and the Gateway's aiortc PeerConnection become the two ends of the media session.

Conceptual peer connection setup

const pc = new RTCPeerConnection({ iceServers: [{ urls: 'stun:stun.l.google.com:19302' }], }); localStream.getTracks().forEach(track => { pc.addTrack(track, localStream); });

At this point there is still no completed network connection. The app has prepared a local media source and a connection object. Negotiation comes next.

DIAGRAM PLACEHOLDER — C-02: Local Track Attached to PeerConnection

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a phone-side diagram: microphone -> MediaStream -> audio MediaStreamTrack -> RTCPeerConnection. Show the STUN server configuration entering RTCPeerConnection. Mark 'No media has reached the Gateway yet.'

# 23. Offer Creation and ICE Gathering

What this section does and why it matters Explain how the phone creates its connection proposal and gathers network candidates.

New technical concepts introduced in this section (in absolute beginner terms) createOffer() generates an SDP proposal based on the current tracks/capabilities. setLocalDescription() adopts the offer and triggers/continues ICE candidate gathering.

Why this matters specifically for RedDust The current simple RedDust signaling model waits for ICE gathering to complete before sending one complete SDP offer rather than implementing trickle ICE.

Simplified non-trickle ICE flow

const offer = await pc.createOffer(); await pc.setLocalDescription(offer); // wait until pc.iceGatheringState === 'complete' const finalOffer = pc.localDescription;

## 23.1 ICE gathering is not ICE connectivity

What this section does and why it matters Prevent confusion between discovering candidate addresses and proving a working path.

New technical concepts introduced in this section (in absolute beginner terms) ICE gathering finds candidates. ICE connection checking tests candidate pairs after both peers know enough about each other.

Why this matters specifically for RedDust Seeing 'ICE gathering complete' is progress, but it does not prove the WebRTC media path is connected.

DIAGRAM PLACEHOLDER — C-03: Gathering versus Checking

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a two-stage diagram: Stage 1 ICE gathering discovers host/srflx/relay candidates; Stage 2 ICE connectivity checks test local/remote candidate pairs after signaling. Put a red warning: 'gathering complete != connection connected'.

# 24. FastAPI /offer Signaling

What this section does and why it matters Explain how the mobile app hands the SDP offer to the Gateway in the current RedDust proof of concept.

New technical concepts introduced in this section (in absolute beginner terms) FastAPI exposes an ordinary HTTP endpoint. JSON carries the SDP string and the SDP type. This is an application-defined signaling mechanism, not the WebRTC media transport.

Why this matters specifically for RedDust Keeping signaling simple lets the team isolate the harder media/connectivity problem. Later signaling can evolve independently.

Conceptual signaling request

POST http://GATEWAY:8000/offer Content-Type: application/json { "sdp": "v=0...", "type": "offer" }

On a local same-Wi-Fi test, GATEWAY is the laptop's LAN IPv4 address, not localhost from the phone. On cloud deployment, it becomes the public/reachable Gateway endpoint and is later expected to move behind production HTTPS/signaling infrastructure.

Important caution HTTP /offer only carries the negotiation description. It does not carry the continuous microphone stream.

DIAGRAM PLACEHOLDER — C-04: HTTP Signaling into FastAPI

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a diagram: React Native TypeScript -> fetch() JSON -> Wi-Fi/IP -> FastAPI POST /offer -> Pydantic Offer model -> aiortc. Draw a separate dashed media lane beneath it to show that continuous audio does not travel through fetch().

# 25. aiortc Answer Creation

What this section does and why it matters Explain the server-side WebRTC peer's reaction to the mobile offer.

New technical concepts introduced in this section (in absolute beginner terms) aiortc implements WebRTC concepts in Python. RTCSessionDescription represents the incoming SDP; RTCPeerConnection owns the remote/local descriptions and the media tracks.

Why this matters specifically for RedDust The Gateway becomes a true WebRTC peer, not merely an HTTP proxy. It terminates the secure media session.

Conceptual aiortc answer flow

pc = RTCPeerConnection() remote_offer = RTCSessionDescription(sdp=offer.sdp, type=offer.type) await pc.setRemoteDescription(remote_offer) answer = await pc.createAnswer() await pc.setLocalDescription(answer) return { 'sdp': pc.localDescription.sdp, 'type': pc.localDescription.type, }

When aiortc applies the remote offer, it learns that the phone intends to send an audio track. Track events allow the Gateway to attach processing or an outbound echo track before final media negotiation is complete.

DIAGRAM PLACEHOLDER — C-05: Gateway Offer Processing

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a Python-side sequence: FastAPI receives Offer JSON -> create aiortc RTCPeerConnection -> register track handler -> setRemoteDescription -> incoming audio track discovered -> optionally add EchoAudioTrack/outbound track -> createAnswer -> setLocalDescription -> return answer JSON.

# 26. Applying the Answer on the Mobile App

What this section does and why it matters Explain the final signaling action on the phone.

New technical concepts introduced in this section (in absolute beginner terms) The JSON answer must be converted into an RTCSessionDescription and applied as the phone's remote description.

Why this matters specifically for RedDust Only after both peers know the offer/answer information can ICE/DTLS complete the actual transport setup.

Apply Gateway answer

const answer = await response.json(); await pc.setRemoteDescription( new RTCSessionDescription(answer) );

A successful setRemoteDescription() proves the signaling description was accepted. It still does not prove the selected ICE path works or that media is flowing.

DIAGRAM PLACEHOLDER — C-06: Phone Applies Gateway Answer

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a before/after peer-connection state diagram. Before: local offer set, answer only received as JSON. After setRemoteDescription: localDescription=offer and remoteDescription=answer, enabling ICE/DTLS connectivity to proceed.

# 27. ICE Checks, DTLS and the Live Media Path

What this section does and why it matters Explain the transition from negotiated descriptions to actual packet flow.

New technical concepts introduced in this section (in absolute beginner terms) ICE connection states describe whether a working candidate pair exists. The PeerConnection state summarizes overall connection progress. Once connectivity is selected, DTLS establishes security and SRTP media can flow.

Why this matters specifically for RedDust For the RedDust echo milestone, 'Connection: connected' plus a received remote audio track is much stronger evidence than a mere HTTP 200 response.

| State family | Typical healthy progression | Meaning |
| --- | --- | --- |
| ICE gathering | new -> gathering -> complete | Candidate discovery finished. |
| ICE connection | new -> checking -> connected/completed | A usable network path has been found/tested. |
| PeerConnection | new -> connecting -> connected | Overall WebRTC transport is live. |

Evidence hierarchy

ICE gathering complete != media connected POST /offer 200 OK != media connected setRemoteDescription OK != media connected Connection: connected = major transport milestone

DIAGRAM PLACEHOLDER — C-07: WebRTC State Progression

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a state-machine infographic with three horizontal tracks: ICE gathering, ICE connection and RTCPeerConnection. Highlight the distinction between gathering complete and connected. Add failure branches for disconnected/failed.

# 28. How the Gateway Receives Kim's Audio

What this section does and why it matters Explain what reaches the Python application after the encrypted network packets are processed by aiortc.

New technical concepts introduced in this section (in absolute beginner terms) aiortc receives SRTP/RTP packets, handles transport/security/codec behavior and exposes an incoming MediaStreamTrack. Calling recv() yields decoded media frames, represented through PyAV/AV frame objects.

Why this matters specifically for RedDust The Gateway application can work at the media-frame boundary without manually implementing SRTP decryption, RTP reassembly or Opus decoding.

Gateway audio-track boundary

@pc.on('track') def on_track(track): if track.kind == 'audio': ... # A processing track can await incoming audio frames: frame = await incoming_track.recv()

This boundary is essential. The network representation and the processing representation are different. On the wire, media is compressed/encrypted/packetised. In Python processing, the application sees decoded audio frames with timing metadata.

DIAGRAM PLACEHOLDER — C-08: Wire Packets to Python AudioFrame

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a layered Gateway diagram: UDP -> SRTP verify/decrypt -> RTP reassembly/timing -> Opus decoder -> aiortc incoming MediaStreamTrack -> PyAV AudioFrame/PCM-like samples -> RedDust application processing. Put WebRTC/aiortc around the first four steps.

# 29. Local Echo as a Proof of the Media Boundary

What this section does and why it matters Explain why the echo test is a powerful milestone before adding conversational AI.

New technical concepts introduced in this section (in absolute beginner terms) An echo track receives Kim's incoming decoded audio frames and returns them as an outbound WebRTC track. It proves both directions of the media pipeline using the same phone and Gateway.

Why this matters specifically for RedDust If echo works, microphone capture, signaling, ICE, DTLS/SRTP, codec negotiation, Gateway receive, Gateway send, mobile receive and speaker playback have all crossed their core boundaries successfully.

Conceptual immediate echo track

class EchoAudioTrack(MediaStreamTrack): kind = 'audio' def __init__(self, incoming_track): super().__init__() self.incoming_track = incoming_track async def recv(self): frame = await self.incoming_track.recv() return frame

The source project later extends this idea by inserting a prerecorded phrase and controlled silence before echoing the caller. That demonstrates that the Gateway can generate/process media while preserving a continuous outbound WebRTC track.

DIAGRAM PLACEHOLDER — C-09: Local Echo Proof

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a circular flow: Kim speaks -> phone mic -> WebRTC -> Gateway incoming audio track -> EchoAudioTrack -> Gateway outbound track -> WebRTC -> phone remote audio -> speaker -> Kim. Label every successfully proven boundary.

# Part IV - Gateway, Syan and the Return Path

The source deck places Syan beyond the RedDust Gateway. The deck explains the conceptual flow - Gateway receives, routes to Syan, receives a response and sends it back - but does not define the exact Gateway-to-Syan protocol. This runbook keeps that boundary explicit.

# 30. RedDust Gateway as the Bridge

What this section does and why it matters Explain the architectural role of the Gateway between the phone and the conversational system.

New technical concepts introduced in this section (in absolute beginner terms) A bridge/edge gateway terminates one protocol boundary and presents media/control information to backend application services. In RedDust, FastAPI handles signaling while aiortc owns the WebRTC peer/media session.

Why this matters specifically for RedDust The Gateway isolates the mobile WebRTC complexity from downstream AI services and gives RedDust one place for media processing, policy, logging and future routing.

Source-deck basis Slide 19 labels the Gateway as 'The Bridge' and lists: receives audio packets, handles signaling, forwards to Syan, receives response and sends back to Kim.

| Gateway responsibility | What it does NOT mean |
| --- | --- |
| Terminate WebRTC peer connection | Syan does not necessarily need to speak WebRTC directly. |
| Handle signaling endpoint | FastAPI does not carry the continuous media bytes through /offer. |
| Expose decoded audio/media frames | Application code need not manually decrypt SRTP or decode Opus. |
| Create outbound track | The mobile app can receive one normal remote WebRTC audio track. |

DIAGRAM PLACEHOLDER — D-01: Gateway as Protocol and Media Boundary

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a three-column diagram: RedDust Mobile (WebRTC) -> RedDust Gateway (FastAPI signaling + aiortc media + frame processing) -> Syan integration boundary. Show separate control and audio arrows and annotate that the Gateway decouples mobile transport from AI backend integration.

# 31. Gateway-to-Syan Integration Boundary

What this section does and why it matters Explain what the source deck establishes and what it leaves unspecified.

New technical concepts introduced in this section (in absolute beginner terms) An architectural boundary identifies responsibility without necessarily defining the exact API yet. The deck says the Gateway forwards Kim's audio to Syan and receives Syan's response, but does not specify the exact network protocol, audio chunk format or streaming API.

Why this matters specifically for RedDust This is an intentional design point for later RedDust work. The WebRTC layer can be proven independently first.

Important caution Do not treat any one Gateway-to-Syan transport as 'defined by the presentation'. The source deck does not specify whether the connection is WebSocket, vendor live-audio API, gRPC, HTTP streaming or another mechanism. Those are implementation decisions outside the deck's evidence.

Whatever transport is chosen later, the engineering contract should explicitly define sample format/rate, chunk duration, timestamps, backpressure, cancellation/barge-in, session identity, error behavior and latency expectations.

| Contract item | Question that must be answered later |
| --- | --- |
| Audio format | PCM? Encoded audio? Which sample format/rate/channels? |
| Streaming transport | WebSocket, vendor live-streaming API, gRPC, etc.? |
| Chunk timing | How many milliseconds of audio per message/frame? |
| Backpressure | What happens if Syan consumes slower than Gateway receives? |
| Turn-taking | How are VAD, interruption and barge-in represented? |
| Return audio | Does Syan stream PCM/encoded audio, and how is timing preserved? |

DIAGRAM PLACEHOLDER — D-02: Gateway-to-Syan Contract Placeholder

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create an interface-contract diagram with Gateway on left and Syan on right. Between them show boxes for transport, audio format, sample rate, chunk duration, timestamps, session ID, backpressure, cancellation/barge-in and errors. Label the exact transport 'TBD by later RedDust design - not defined in source deck'.

# 32. Syan Hears, Understands and Responds

What this section does and why it matters Preserve the source deck's high-level conversational processing model without pretending it specifies internal AI implementation.

New technical concepts introduced in this section (in absolute beginner terms) At the conceptual level, Syan receives audio input, performs AI processing/understanding and produces audio output. The internal model/VAD/tool-calling details are separate from the WebRTC transport described here.

Why this matters specifically for RedDust The clean boundary lets the voice transport team prove audio reliability before coupling it to higher-level conversational intelligence.

Source-deck basis Slide 20 shows a three-stage conceptual Syan flow: Hears Kim (Audio Input) -> Understands (AI Processing) -> Responds (Audio Output).

From a real-time systems perspective, Syan's output must eventually be delivered as a timed audio stream. The Gateway should not wait for an entire long response file if the product goal is natural, low-latency conversation; a future streaming design should preserve incremental audio timing.

DIAGRAM PLACEHOLDER — D-03: Syan Processing Boundary

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a diagram titled 'Syan Hears and Responds'. Show Gateway audio stream -> Syan audio input -> AI understanding -> response generation -> streaming audio output -> Gateway. Keep the internals generic and label them 'AI processing - exact implementation outside this deck'.

# 33. Building the Outbound Audio Track

What this section does and why it matters Explain how backend-generated response audio becomes something the phone can receive as normal WebRTC media.

New technical concepts introduced in this section (in absolute beginner terms) A WebRTC sender needs a MediaStreamTrack that produces correctly timed audio frames. The track's recv() method must emit frames on a continuous timeline; the WebRTC stack then handles encoding, RTP, SRTP and transport.

Why this matters specifically for RedDust This is the same mechanism used by EchoAudioTrack and can later carry Syan-generated audio instead of caller echo.

Timing is important. Returning many queued frames instantly would create a burst rather than real-time playback. Frames need coherent timestamps (PTS/time base) and pacing so the sender emits them at the intended media rate.

## 33.1 Why a simple sleep can be misleading

What this section does and why it matters Explain the difference between pausing application logic and maintaining a live media timeline.

New technical concepts introduced in this section (in absolute beginner terms) A WebRTC audio sender expects a continuing media clock. For deliberate silence, a robust media track may emit silence frames with valid timestamps rather than simply stop producing frames for an arbitrary period.

Why this matters specifically for RedDust The Step 19B experiment in the earlier project intentionally demonstrates prefix audio, timed silence and then echo on one continuous outbound track.

DIAGRAM PLACEHOLDER — D-04: Outbound Media Track Timeline

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a horizontal timeline for a backend-generated MediaStreamTrack: response prefix frames -> timed silence frames -> streaming Syan/echo frames. Show PTS/time-base continuity and then WebRTC encoding/packetisation downstream.

# 34. The Return Network Path

What this section does and why it matters Walk through the reverse of Kim's inbound path.

New technical concepts introduced in this section (in absolute beginner terms) Syan/Gateway response audio becomes outbound media frames; WebRTC encodes, packetises and encrypts them; packets cross the selected ICE route; the mobile WebRTC stack reverses the process.

Why this matters specifically for RedDust Bidirectional real-time communication is two concurrent media directions sharing one negotiated PeerConnection, not a request/response file download.

Source-deck basis Slide 21 shows Syan's audio -> packetisation -> WebRTC engine -> RedDust Gateway -> Kim. The runbook expands that into the codec/RTP/SRTP/jitter-buffer steps that the slide abstracts away.

Return media pipeline

Gateway audio frames -> Opus encode -> RTP timestamp/sequence -> SRTP protect -> UDP/IP over selected ICE path -> mobile SRTP/RTP receive -> jitter buffer -> Opus decode -> remote audio track -> phone audio output

DIAGRAM PLACEHOLDER — D-05: Return Path to Kim

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a detailed return-path diagram from Syan response audio through Gateway outbound track, Opus, RTP/SRTP, ICE-selected network path, mobile jitter buffer, Opus decoder, remote audio track, OS audio mixer/DAC/amplifier and speaker to Kim.

# 35. Mobile Receive, Jitter Buffer, Decode and Speaker Playback

What this section does and why it matters Explain why the React Native application normally does not need to decrypt packets, decode Opus or manually push PCM bytes to the speaker.

New technical concepts introduced in this section (in absolute beginner terms) The native WebRTC engine converts the network stream into a remote MediaStreamTrack. Its internal receive pipeline deals with packet timing, security and codec decoding; the mobile audio subsystem routes the result to an output device.

Why this matters specifically for RedDust RedDust application code should operate at the track/session/routing level unless a specific custom DSP requirement justifies going lower.

In a call-style app, the final audio route can be earpiece, loudspeaker, wired headset or Bluetooth. The RedDust Step 19A experiment separately tests loudspeaker/earpiece routing so a routing problem is not mistaken for a WebRTC media problem.

| Layer | Who normally handles it |
| --- | --- |
| SRTP verification/decryption | Native WebRTC engine |
| RTP sequence/timestamp handling | Native WebRTC engine |
| Jitter buffering | Native WebRTC engine |
| Opus decoding | Native WebRTC engine |
| Remote MediaStreamTrack event | react-native-webrtc exposes it to app |
| Speaker/earpiece routing | Mobile OS/native audio routing; application may select policy |

DIAGRAM PLACEHOLDER — D-06: Mobile Receive Stack

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a layer stack inside the phone: Wi-Fi/5G packet arrival -> UDP -> SRTP/RTP -> jitter buffer -> Opus decode -> remote MediaStreamTrack -> Android/iOS audio subsystem -> DAC/amplifier -> loudspeaker/earpiece. Put a bracket around the native WebRTC-managed layers.

# Part V - Engineering Calculations, Latency and Troubleshooting

This part turns the concepts into practical calculations and debugging methods that interns can use when something sounds wrong or does not connect.

# 36. Sampling and Frame Mathematics

What this section does and why it matters Give interns enough mathematics to reason about audio timing without requiring a signal-processing degree.

New technical concepts introduced in this section (in absolute beginner terms) Sample rate tells samples per second; frame duration tells how many seconds a frame represents; multiplying them gives samples per frame.

Why this matters specifically for RedDust These calculations appear when sizing buffers, resampling, generating silence and validating timestamps.

Frame-size examples

samples_per_frame = sample_rate * frame_duration_seconds At 48 kHz and 20 ms: 48,000 * 0.020 = 960 samples per channel At 48 kHz and 10 ms: 48,000 * 0.010 = 480 samples per channel

| Sample rate | Frame duration | Samples per channel |
| --- | --- | --- |
| 48,000 Hz | 10 ms | 480 |
| 48,000 Hz | 20 ms | 960 |
| 48,000 Hz | 40 ms | 1,920 |
| 44,100 Hz | 20 ms | 882 |

DIAGRAM PLACEHOLDER — E-01: Frame Math Timeline

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a 1-second timeline at 48 kHz, subdivided into 20 ms frames. Show 50 frames per second and 960 samples per channel in each frame. Include the formula samples = sample_rate * seconds.

# 37. Bitrate and Bandwidth Calculations

What this section does and why it matters Separate uncompressed PCM bitrate from compressed network bitrate.

New technical concepts introduced in this section (in absolute beginner terms) PCM bitrate follows sample_rate * bits_per_sample * channels. Codec bitrate is selected/controlled by the codec and can be far lower. Network bandwidth also includes RTP/SRTP/UDP/IP overhead.

Why this matters specifically for RedDust Capacity planning and troubleshooting must use the correct layer's bitrate; otherwise estimates can be wrong by an order of magnitude.

Uncompressed examples

PCM bitrate = sample_rate * bit_depth * channels 48,000 * 16 * 1 = 768,000 bits/s = 768 kbps (mono PCM) 44,100 * 16 * 2 = 1,411,200 bits/s = 1,411.2 kbps (stereo PCM)

An Opus voice stream may use a much lower configured/effective payload bitrate than raw PCM. The actual wire bitrate is payload plus headers and protocol overhead. Treat the exact negotiated/configured bitrate as runtime configuration rather than assuming the PCM formula applies to WebRTC packets.

DIAGRAM PLACEHOLDER — E-02: PCM versus Network Bitrate

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a comparative bandwidth diagram: raw mono 48 kHz/16-bit PCM at 768 kbps -> Opus compression -> much smaller variable/selected media bitrate -> add RTP/SRTP/UDP/IP overhead -> actual wire bitrate. Emphasize that PCM bitrate and codec bitrate are different.

# 38. Latency Budget for Natural Conversation

What this section does and why it matters Explain why 'real time' is the sum of many small delays rather than one network number.

New technical concepts introduced in this section (in absolute beginner terms) End-to-end latency includes capture framing, audio processing, codec lookahead/frame duration, packet queueing, network transit, jitter buffering, Gateway processing, AI response generation and the reverse path.

Why this matters specifically for RedDust RedDust's product experience depends on the total conversational delay, so each layer should avoid unnecessary buffering and blocking.

| Latency contributor | Examples |
| --- | --- |
| Capture | Audio callback/frame accumulation. |
| Codec | Frame duration and algorithmic delay. |
| Network | Wi-Fi/5G routing and congestion. |
| Jitter buffer | Intentional receive buffering. |
| Gateway | Frame processing, queueing, resampling. |
| Syan | Inference/turn processing and first-audio generation. |
| Playback | Decode, audio device buffer and route. |

A latency budget is therefore best treated as a measured timeline. Log timestamps at meaningful boundaries rather than guessing from subjective impressions.

DIAGRAM PLACEHOLDER — E-03: End-to-End Latency Waterfall

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a latency waterfall from Kim speaking to hearing Syan: capture frame -> encode -> outbound network -> Gateway receive -> AI first-response time -> Gateway send -> return network -> jitter buffer/decode -> speaker. Use empty numeric boxes for teams to fill with measured milliseconds later.

# 39. Layer-by-Layer Troubleshooting

What this section does and why it matters Give interns a disciplined order for diagnosing failures rather than changing many layers at once.

New technical concepts introduced in this section (in absolute beginner terms) Layered troubleshooting means proving the lowest dependent layer first. A healthy HTTP endpoint does not prove ICE; a connected PeerConnection does not prove a remote audio track; a remote track does not prove correct speaker routing.

Why this matters specifically for RedDust This approach directly supports the staged RedDust milestones from microphone capture to echo to Internet deployment.

| Observed symptom | First layer to inspect | Useful evidence |
| --- | --- | --- |
| No microphone track | Device permission / capture | getUserMedia result; local track state. |
| /offer fails | Signaling / IP / firewall | HTTP status, FastAPI logs, device can reach Gateway address. |
| Answer received but connection fails | ICE/NAT/firewall | ICE states, candidates, selected pair, STUN/TURN availability. |
| Connected but Gateway gets no audio | Track/media negotiation | Gateway track event, SDP direction/codecs, aiortc logs. |
| Gateway sends track but phone logs none | Outbound media negotiation | Answer direction, phone track event. |
| Remote track exists but silence | Audio content/timing/routing | Frame levels, timestamps, speaker route, mute state. |
| Choppy audio | Jitter/loss/timing | WebRTC stats, packet loss, jitter, buffer behavior. |
| Metallic/garbled audio | Codec/format/timing or upstream capture | Negotiated codec, resampling/sample-format correctness; distinguish from aliasing. |

DIAGRAM PLACEHOLDER — E-04: Troubleshooting Ladder

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a vertical troubleshooting ladder: 1 permissions/capture, 2 local track, 3 signaling, 4 SDP, 5 ICE, 6 DTLS/SRTP, 7 Gateway track, 8 outbound track, 9 mobile remote track, 10 audio routing. For each rung show a pass/fail evidence item.

# 40. Security and Privacy Boundaries

What this section does and why it matters Explain what WebRTC protects and what still requires application-level security design.

New technical concepts introduced in this section (in absolute beginner terms) WebRTC protects media in transit using DTLS/SRTP. That does not automatically define authorization, identity, recording policy, data retention, AI privacy or server-side access control.

Why this matters specifically for RedDust RedDust deals with intimate voice interactions, so transport encryption is necessary but not sufficient.

• Use HTTPS/WSS for production signaling and authenticated session establishment.

• Do not log complete SDP or access tokens casually in production; SDP can expose network information and temporary credentials.

• Limit recording and transcript retention to explicit product/security policy.

• Protect Gateway-to-Syan transport and credentials separately from WebRTC transport security.

• Use least-privilege cloud identities and secret-management systems for backend services.

• Treat WebRTC media encryption and end-to-end product privacy as related but distinct concepts.

DIAGRAM PLACEHOLDER — E-05: Security Boundaries

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a layered security diagram: user/device permission -> authenticated signaling over HTTPS -> ICE/DTLS/SRTP media -> Gateway service identity -> protected Gateway-to-Syan channel -> data/storage policy. Add banners: 'SRTP protects media in transit' and 'Transport encryption does not replace authorization/privacy policy'.

# 41. Testing Strategy for RedDust Voice

What this section does and why it matters Organize testing so each new milestone proves one additional boundary.

New technical concepts introduced in this section (in absolute beginner terms) A staged test matrix isolates hardware, software and network variables. Start locally, then increase realism only after the lower layer is proven.

Why this matters specifically for RedDust This is the philosophy behind the project's microphone test, same-Wi-Fi echo, audio-route test, backend media insertion and later Internet/GCP test.

| Stage | What it proves | Do not infer yet |
| --- | --- | --- |
| Microphone only | Permission + native capture + local audio track. | No Gateway/network proof. |
| Offer/answer | Signaling + SDP compatibility. | No media connectivity proof. |
| ICE/PeerConnection connected | Working secure transport path. | No return-audio proof. |
| Remote track received | Gateway outbound media negotiated and delivered. | Speaker route may still be wrong. |
| Audible local echo | Bidirectional media + playback work on LAN. | Internet NAT/TURN not yet proven. |
| Backend prefix/delay/echo | Gateway can generate/process/timestamp media. | AI integration not yet proven. |
| Internet/mobile network | Cloud firewall/NAT/ICE path works. | Production TURN/SLO/security not yet complete. |

DIAGRAM PLACEHOLDER — E-06: Progressive Voice Test Pyramid

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a pyramid or staircase: microphone -> signaling -> ICE connected -> remote track -> audible echo -> audio routing -> backend media processing -> Internet echo -> Syan live integration. Each step should list the new boundary proven.

# 42. Final End-to-End Mental Model

What this section does and why it matters Combine the whole document into one mental model that an intern can remember.

New technical concepts introduced in this section (in absolute beginner terms) The same voice changes representation many times: pressure wave, electrical voltage, numeric samples, compressed codec frames, encrypted network packets, decoded media frames, AI input/output, then the reverse path to physical sound.

Why this matters specifically for RedDust Most engineering mistakes happen when two adjacent representations are confused - for example WAV with PCM, PCM with Opus, signaling with media, ICE gathering with ICE connectivity, or a remote track with actual speaker routing.

One-page conceptual chain

PHYSICAL Kim's voice / air pressure -> microphone voltage DIGITAL CAPTURE -> sampled + quantised audio -> PCM/audio frames WEBRTC SEND -> audio processing / Opus -> RTP -> SRTP -> UDP/IP -> ICE-selected network path GATEWAY -> aiortc decrypt/reassemble/decode -> incoming audio track / AudioFrame -> RedDust/Syan processing boundary -> outbound audio track WEBRTC RETURN -> Opus -> RTP/SRTP -> network -> jitter buffer -> decode -> remote audio track PHYSICAL OUTPUT -> mobile audio subsystem / DAC / amplifier -> speaker -> Kim hears response

DIAGRAM PLACEHOLDER — E-07: Complete RedDust Voice Stack

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create the most comprehensive diagram in the document. Use six horizontal layers: Physical Acoustics, Phone Electronics/ADC, Mobile WebRTC, Network, RedDust Gateway, Syan/Application, then mirror the return path. Label every representation transition and clearly separate signaling from media. Make it suitable as a team onboarding wall chart.

# Appendix A - Absolute-Beginner Glossary

What this section does and why it matters Provide one place to look up every important acronym and concept.

New technical concepts introduced in this section (in absolute beginner terms) The glossary uses project-specific plain-English definitions rather than standards-language definitions.

Why this matters specifically for RedDust Interns should use this appendix while reading logs, SDP, WebRTC documentation and Gateway code.

| Term | Plain-English meaning |
| --- | --- |
| Acoustic wave | Changing air pressure carrying sound. |
| ADC | Analog-to-Digital Converter; converts conditioned analog voltage into digital sample values. |
| Aliasing | False lower-frequency content created when frequencies above the safe sampling limit are sampled incorrectly. |
| Amplitude | Strength/size of a signal at an instant. |
| Anti-aliasing filter | Analog low-pass filter before ADC that removes unsafe high frequencies. |
| Bit depth | Number of bits available to represent each PCM sample's amplitude. |
| Buffer | Memory holding audio samples/frames temporarily. |
| Codec | Encoder/decoder that compresses and reconstructs media. |
| DTLS | Datagram Transport Layer Security; secure handshake/keying used by WebRTC. |
| Frame | A time-bounded block of audio samples. |
| ICE | Interactive Connectivity Establishment; gathers/tests network candidates. |
| Jitter | Variation in packet arrival timing. |
| MediaStream | WebRTC container grouping media tracks. |
| MediaStreamTrack | A live audio or video source/sink track. |
| NAT | Network Address Translation; maps private network addresses to public mappings. |
| Nyquist frequency | Half the sampling rate; highest band edge that can be represented under the theorem's conditions. |
| Opus | Low-latency audio codec commonly used by WebRTC. |
| PCM | Pulse Code Modulation; numerical sample representation of audio. |
| Quantisation | Mapping a measured sample amplitude to one of finitely many numeric levels. |
| RTP | Real-time Transport Protocol; timestamps/sequences real-time media. |
| RTCPeerConnection | WebRTC object representing a peer media connection. |
| Sampling rate | Number of samples measured per second. |
| SDP | Session Description Protocol; text description used during offer/answer negotiation. |
| Signaling | Application-defined exchange of WebRTC session descriptions/control information. |
| SRTP | Secure Real-time Transport Protocol; encrypted/authenticated RTP media. |
| STUN | Session Traversal Utilities for NAT; helps discover public-facing mappings. |
| TURN | Traversal Using Relays around NAT; relays media when direct paths fail. |
| UDP | User Datagram Protocol; low-overhead datagram transport used by most WebRTC media paths. |
| WAV | Audio file container commonly used to store uncompressed PCM plus format metadata. |
| WebRTC | Real-time communication system/API family combining media tracks, codecs, connectivity, security and transport behavior. |

# Appendix B - Formula and Calculation Sheet

What this section does and why it matters Collect the formulas used most often in this runbook.

New technical concepts introduced in this section (in absolute beginner terms) These formulas are simple dimensional relationships. Always keep units visible while calculating.

Why this matters specifically for RedDust They help interns sanity-check frame sizes, sample timing and raw PCM data rates.

Core formulas

Sampling interval: Ts = 1 / fs Nyquist requirement (ideal band-limited case): fs > 2 * fmax Nyquist frequency: fN = fs / 2 Quantisation levels: levels = 2^N PCM bitrate: bitrate = sample_rate * bits_per_sample * channels Samples in one frame: samples = sample_rate * frame_duration_seconds Frames per second: frames_per_second = 1 / frame_duration_seconds

Worked examples

48 kHz, 20 ms mono frame: 48,000 * 0.020 = 960 samples 48 kHz, 16-bit mono PCM bitrate: 48,000 * 16 * 1 = 768,000 bits/s 44.1 kHz, 16-bit stereo PCM bitrate: 44,100 * 16 * 2 = 1,411,200 bits/s

DIAGRAM PLACEHOLDER — F-01: Formula Cheat Sheet

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a single-page infographic cheat sheet with the formulas Ts=1/fs, fs>2fmax, Nyquist=fs/2, levels=2^N, PCM bitrate=sample_rate*bit_depth*channels, and samples_per_frame=sample_rate*seconds. Include 48 kHz/20 ms and 44.1 kHz/16-bit stereo worked examples.

# Appendix C - Diagnostic Checklist

What this section does and why it matters Provide a practical checklist to use during RedDust voice debugging.

New technical concepts introduced in this section (in absolute beginner terms) Each checkbox corresponds to a boundary proved independently by the project milestones.

Why this matters specifically for RedDust The rule is: do not jump upward until the lower layer is proven.

| Done | Check |
| --- | --- |
| ☐ | Phone microphone permission granted. |
| ☐ | getUserMedia returns a live local audio track. |
| ☐ | Local track is attached to RTCPeerConnection. |
| ☐ | Offer is created and set as local description. |
| ☐ | ICE gathering completes or trickle ICE is functioning. |
| ☐ | Phone can reach Gateway signaling endpoint. |
| ☐ | FastAPI /offer returns success and a valid answer. |
| ☐ | Phone applies answer as remote description. |
| ☐ | ICE connection reaches connected/completed. |
| ☐ | PeerConnection reaches connected. |
| ☐ | Gateway track handler receives an audio track. |
| ☐ | Gateway can recv decoded audio frames. |
| ☐ | Gateway adds/produces an outbound audio track. |
| ☐ | Phone receives a remote audio track. |
| ☐ | Remote audio track has live audio content. |
| ☐ | Phone output route is correct (speaker/earpiece/Bluetooth as intended). |
| ☐ | Audible local echo works. |
| ☐ | Backend-generated prefix/silence/echo sequencing works if that test is enabled. |
| ☐ | Internet test repeats successfully outside same Wi-Fi. |
| ☐ | TURN behavior is tested for restrictive networks before production. |

# Appendix D - Diagram Production Index

What this section does and why it matters Provide a planning index for the future diagrams already specified throughout the document.

New technical concepts introduced in this section (in absolute beginner terms) Each placeholder contains a future ChatGPT image-generation prompt, so diagrams can be generated one by one without rewriting the architecture text.

Why this matters specifically for RedDust Generate diagrams only after the technical text is reviewed; that prevents polished visuals from freezing an incorrect architecture too early.

| Diagram ID | Future diagram topic |
| --- | --- |
| A-01 | End-to-End Voice Journey |
| A-02 | Analog Speech Wave |
| A-03 | Microphone as a Transducer |
| A-04 | ADC Pipeline |
| A-05 | Sampling in the Time Domain |
| A-06 | Nyquist Frequency and Audio Band |
| A-07 | Aliasing Before and After |
| A-08 | 44.1 kHz versus 48 kHz Boundaries |
| A-09 | Quantisation Grid |
| A-10 | Bit Depth and Dynamic Range |
| A-11 | Quantised Samples to PCM |
| A-12 | PCM versus WAV |
| B-01 | Samples to Frames to Buffers |
| B-02 | PCM to Opus and Back |
| B-03 | WebRTC Media Packet Stack |
| B-04 | Jitter Buffer |
| B-05 | WebRTC Engine Responsibilities |
| B-06 | Signaling Path versus Media Path |
| B-07 | SDP Offer/Answer |
| B-08 | ICE/STUN/TURN Candidate Discovery |
| B-09 | DTLS-SRTP Security |
| C-01 | Mobile Audio Capture Boundary |
| C-02 | Local Track Attached to PeerConnection |
| C-03 | Gathering versus Checking |
| C-04 | HTTP Signaling into FastAPI |
| C-05 | Gateway Offer Processing |
| C-06 | Phone Applies Gateway Answer |
| C-07 | WebRTC State Progression |
| C-08 | Wire Packets to Python AudioFrame |
| C-09 | Local Echo Proof |
| D-01 | Gateway as Protocol and Media Boundary |
| D-02 | Gateway-to-Syan Contract Placeholder |
| D-03 | Syan Processing Boundary |
| D-04 | Outbound Media Track Timeline |
| D-05 | Return Path to Kim |
| D-06 | Mobile Receive Stack |
| E-01 | Frame Math Timeline |
| E-02 | PCM versus Network Bitrate |
| E-03 | End-to-End Latency Waterfall |
| E-04 | Troubleshooting Ladder |
| E-05 | Security Boundaries |
| E-06 | Progressive Voice Test Pyramid |
| E-07 | Complete RedDust Voice Stack |
| F-01 | Formula Cheat Sheet |

# Closing Summary

What this section does and why it matters Close the training document with the minimum set of ideas an intern should retain.

New technical concepts introduced in this section (in absolute beginner terms) The central lesson is representation and responsibility: understand what form the voice is in at each boundary and which component owns the next transformation.

Why this matters specifically for RedDust Once this model is clear, RedDust's echo test, cloud deployment, TURN integration and later Syan streaming work become extensions of one coherent real-time media pipeline rather than unrelated technologies.

• Sound begins as a continuous pressure wave and becomes an analog electrical signal in the microphone.

• Sampling discretises time; quantisation discretises amplitude; PCM represents the resulting numerical samples.

• WAV is a file container; WebRTC live media is not a WAV-file transfer.

• WebRTC usually compresses audio with Opus, timestamps/sequences it with RTP, protects it with SRTP and transports it over an ICE-selected network path.

• Signaling establishes the session; media carries the voice.

• FastAPI handles RedDust's current simple signaling endpoint; aiortc is the WebRTC media peer on the Gateway.

• The Gateway application normally receives decoded media frames, not encrypted SRTP bytes.

• A remote audio track on the phone is already past decryption, RTP handling and codec decoding in the native WebRTC stack.

• The exact Gateway-to-Syan streaming contract is a later architecture decision not specified by the source deck.

• Debug one boundary at a time: capture -> signaling -> ICE -> transport -> Gateway track -> outbound track -> mobile remote track -> speaker routing.

DIAGRAM PLACEHOLDER — F-02: One-Page RedDust Voice Engineering Summary

RESERVED SPACE FOR FUTURE TECHNICAL / ONBOARDING DIAGRAM Future ChatGPT image-generation prompt Create a final one-page onboarding infographic summarizing the entire voice journey: Kim -> acoustics -> microphone/electronics -> ADC -> PCM -> WebRTC/Opus/RTP/SRTP -> ICE-selected network -> Gateway/aiortc -> Syan boundary -> outbound track -> return WebRTC -> mobile speaker. Include small boxes for the three most common confusions: PCM vs WAV, signaling vs media, ICE gathering vs ICE connected.